Design Requirements

Impedance Control

Impedance control is required. Impedance control is required in the following layers

- 50 Ohms (+-10%) Single-Ended Microstrip

  - L1 (F_Cu) ref. L2 (In1_Cu)

  - Trace width = 0.23

- 50 Ohms (+-10%) Single-Ended Microstrip

  - L6 (In5_Cu) ref. L7 (In6_Cu) and L5 (In4_Cu)

  - Trace width = 0.14

- 50 Ohms (+-10%) Single-Ended Microstrip

  - L8 (B_Cu) ref. L7 (In6_Cu)

  - Trace width = 0.23

- 90 Ohms (+-10%) Differential Microstrip (50 Ohms (+-10%) Single-Ended)

  - L1 (F_Cu) ref. L2 (In1_Cu)

  - Trace width = 0.23 mm

  - Trace spacing = 0.26mm

  - USB Lines

- 100 Ohms (+-10%) Differential Microstrip (50 Ohms (+-10%) Single-Ended)

  - L1 (F_Cu) ref. L2 (In1_Cu)

  - Trace width = 0.23 mm

  - Trace spacing = 1.0mm

  - Ethernet lines to MagJack

Ethernet Design

Considerations in this design are listed below,

- RMII 10/100 Mbps ethernet design is used.

- Matched net lengths are required

  - RXD0, RXD1, and CRS_DV are matched with each other

  - TXD0, TXD1, and TX_EN are matched with each other

- Differential lines [TXN, TXP] and [RXN, RXP] are intra-pair length matched.

- Impedance control is required (See Section Impedance Control section)

  - 50 Ohm Single Ended microstrip

    - Trace width = 0.23mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

    - Trace width = 0.14mm

      - L6 (In5_Cu) ref. L7 (In6_Cu) and L5 (In4_Cu)

  - 100 Ohm Differential microstrip

    - Trace width = 0.23 mm and Trace spacing = 1.0mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

eMMC Memory Design

Considerations in this design are listed below,

- Laser Micro-vias from Layer 1 to Layer 6 is used as via in-pad

- The data lines are length matched – [D0-D7] to each other and to the CMD line.

- Impedance control is required

  - 50 Ohm Single Ended microstrip

    - Trace width = 0.23mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

    - Trace width = 0.14mm

      - L6 (In5_Cu) ref. L7 (In6_Cu) and L5 (In4_Cu)

SD Card Design

Considerations in this design are listed below,

- The data lines are length matched – [D0-D3] to each other and to the CMD line.

- Impedance control is required

  - 50 Ohm Single Ended microstrip

    - Trace width = 0.23mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

    - Trace width = 0.14mm

      - L6 (In5_Cu) ref. L7 (In6_Cu) and L5 (In4_Cu)

USB Design

Considerations in this design are listed below,

- Impedance control is required

  - 50 Ohm Single Ended microstrip

    - Trace width = 0.23mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

    - Trace width = 0.14mm

      - L6 (In5_Cu) ref. L7 (In6_Cu) and L5 (In4_Cu)

  - 90 Ohm Differential microstrip

    - Trace width = 0.23 mm and Trace spacing = 0.26mm

      - L1 (F_Cu) ref. L2 (In1_Cu)

General

- Solder bridge is required. Some of them are bridged and some are left open. The ones that are bridged need to be bridged via solder during assembly are given below:

  - JP401, JP402, JP403, JP406, and JP408